## Using the AutoSegmentator

### 1. Prepare the CT

- Extract your CT ZIP file (if compressed).

- Copy or move the extracted CT folder into:

AutoSegmentator/Input/

(You may keep it elsewhere if you prefer, but placing it inside the Input folder makes browsing easier.)

### 2. Launch the application

Run:

python main.py

or simply double-click main.py if Python is associated with .py files.

The AutoSegmentator interface will open.

### 3. Select the CT folder

At the top of the application:

- Browse to your CT folder.

- Or manually enter the folder path.

### 4. Choose the output folder

The default output folder is recommended:

AutoSegmentator/Output

You may choose a different folder if desired.

### 5. Select structures

Expand the anatomical regions and select the structures you wish to segment.

You may:

- select an entire region

- select only specific organs

- combine structures from multiple regions

Only the selected structures will be included in the final RT Structure Set.

### 6. GPU / CPU

If your computer has an NVIDIA GPU with CUDA installed:

☑ Enable Use GPU Acceleration

Otherwise leave it unchecked and the segmentation will run on the CPU.

### 7. Run the segmentation

Click Predict.

The application will:

- run TotalSegmentator

- generate the requested .nii.gz segmentation masks

- automatically convert them into an RT Structure Set (RS.dcm)

Depending on your hardware and the number of selected structures, this may take several minutes.

### 8. Output

The output folder will contain:

- segmented .nii.gz masks

- RS.dcm (the generated RT Structure Set)

Import RS.dcm into Eclipse for review.

## Running another case

When processing a new patient:

- Select the new CT folder.

- Click Predict again.

The program will automatically:

- delete old .nii.gz segmentation files

- generate the new segmentations

Note: Existing RT Structure Set files (RS.dcm) are not automatically deleted. If you are using the same output folder, delete or rename the previous RS.dcm before running another case to avoid confusion.

## Notes

- Only the structures you selected in the interface are converted into the RT Structure Set.

- Colors are assigned automatically according to the predefined color table in convert.py.

- Certain structures (such as the left and right lungs) are automatically merged into a single ROI where appropriate.

- The generated RT Structure Set is intended for review and should always be verified before clinical use.